Comments on Drawings Issued April 1, 2008

Drawing List (per City of Orinda Design Review Application)

- Vicinity Map (include neighborhood data table)

- Site Plan (1"= 20') (This possibly can include roof plan, and should include project data table)

- Conceptual Grading Plan  (1"= 20') (can be included in site plan if minor, and can also include drainage plan)

- Roof Plan  (1"= 20') (can be superimposed over grading plan – Note: this seems odd)

- Landscape Plan (1"= 20')  (For smaller projects OK to add to site plan)

- Floor Plans – two req'd (1/4" = 1'-0")

- Building Elevations – two req'd (1/4" = 1'-0")

- Building Cross Sections – three req'd (1/4" = 1'-0")

- Full E/W Property Cross Section – one req'd (1/8" = 1'-0")

Dwg I – Vicinity Map

- Index Dwg # 10  - Should be Sheet A4.2

- Show distance to No. 20 La Vuelta

- Property data table

- Proposed adjusted floor area should be 3283

- Max permissible should read 3328 s.f., not 3338

Dwg A1r – Site Plan with Roof Plan

- Route main corrugated drain pipe so as not to conflict with house.

- All landscaping is now gone.  The intent was to delete only the landscape legend and new landscaping and to leave the existing trees shown.  Of course it is clearer without the trees, so we may want to leave it like it is.

- Note roof ridge elevations

- Note typical roof pitch

Dwg G1 – Grading Plan

- Cuts don't appear correct.  Don’t we need at least 30" below finish floor.  Where finish floor is +87, we have to excavate to 84'-6".  Where finish floor is +91, we have to excavate to 88'-6"

- Lines depicting upper floor and deck above are still confusing.

- Clarify text of note: 2nd para, 4th line – "already up to 15' back" should read "already 20' back".

- Correct spelling in text of note: 4th  para, 3rd line – "wil" should read "will".

- Move landscape note (the one above the north arrow) to landscape page.

Dwg L1 – Landscape Plan

- Correct spelling of arborist address (Freeman Road)

- Correct certification No. of arborist (#206-B)

- Correct spelling in landscape notes: 1st line, 2nd para should read "… as follows:"

Dwg A2u – Upper Floor Plan

- Extend pool to column

Dwg A2g – Ground Floor Plan

- North arrow

- Clarify upper floor areas above, e.g. driveway and entry deck

Dwg A3.1 – N and W Elevations

- Make dimensions readable

- Grid lines in west elevation are redundant and incorrectly shown

- Roof elevations should be noted in decimal or ft/in format, not both.  Also inconsistencies in actual elevations indicated.

Dwg A3.2 – E and S Elevations

- Fencing and roadway is confusing.

- Roof elevations should be noted in decimal or ft/in format, not both.  Also inconsistencies in actual elevations indicated.

- Height in east elev. should be 34'.

Dwg A4.1 – Section A-A   (1/4" = 1'-0")

- Overall height dimension should be 34'

- Roof elevations should be noted in decimal or ft/in format, not both.  Also inconsistencies in actual elevations indicated.

Dwg A4.2 – Section B-B  (1/8" = 1'-0")

- Overall height dimension should be 34'

Dwg A4.3 – Section C-C  (1/4" = 1'-0")

- Roof elevations should be noted in decimal or ft/in format, not both.  Also inconsistencies in actual elevations indicated.

Dwg A4.4 – Section D-D  (1/4" = 1'-0")

- Show legends

- Show graphical scale

- Remove trees where not needed

- Confirm garage roof elev – 115.4' or 115'-4".