Comments on Drawings Issued March 26, 2008

Drawing List (per City of Orinda Design Review Application)

- Vicinity Map (include neighborhood data table)

- Site Plan (1"= 20') (This possibly can include roof plan, and should include project data table)

- Conceptual Grading Plan  (1"= 20') (can be included in site plan if minor, and can also include drainage plan)

- Roof Plan  (1"= 20') (can be superimposed over grading plan – Note: this seems odd)

- Landscape Plan (1"= 20')  (For smaller projects OK to add to site plan)

- Floor Plans – two req'd (1/4" = 1'-0")

- Building Elevations – two req'd (1/4" = 1'-0")

- Building Cross Sections – three req'd (1/4" = 1'-0")

- Full E/W Property Cross Section – one req'd (1/8" = 1'-0")

General Notes

- Drawings would look better if fonts were resized and made more consistent through the drawings.  Some are too large, and many are of inconsistent size.

Dwg I – Vicinity Map

- Update the index with correct drawing order and scales

- Arborist address "Freemman Road"  should be "Freeman Road".

- Arborist certification No. is 206-B

- Property data table

- Column "Floor Area" should read "Adjusted Floor Area"

Dwg A1r – Site Plan with Roof Plan

- Use consistent drawing title, i.e., "Site Plan with Roof Plan".

- Scale is 1" = 10'

- Show drainage features.

- North culvert to ditch via corrugated pipe

- Roof and deck drains directed to the corrugated pipe (ref above) or to the "creek".

- Indicate parking.

- Indicate fences.

- Indicate impervious surfaces.

- Provide Project Data Table with:

- Floor area calcs and FAR

- Parcel net area

- Itemized calculation of impervious surfaces and square footage.

- Fix extent of lap pool.

- Note average slope of site (suggest 40% which is the same as the 4.8:12 roof pitch)

- Clean up the dimensions.

- Eliminate unnecessary dimensions, section cuts, grid lines and notes.

- Dimensions that remain should be more readable, e.g. the 72' dimension to the SE corner of the garage conflicts with the 50' creek setback dimension.

- Delete ridge skylights at master bath, kitchen, and garage.

- Show skylight in guest bath.

- Show roof pitches.  Since all roofs have same 4.8:12 pitch, it is only necessary to show once (or maybe twice) and call out as "typical".

- Label roof as "Slate Type".

- Delete landscape legend and new landscaping and refer to the landscape page

- Consider modifying creek setback by adding a south line that mirrors the north line.  The "creek" is then formed by a triangular area with the apex at the outlet of the culvert.

Dwg G1 – Grading Plan

- Title – "Grading Plan"

- Show 1 ft contours.  (City requires a min contour interval of 2 ft)

- Delete landscape legend and notes, and new landscaping and refer to the landscape page for this information.

- Indicate that there are no known geologic or potentially hazardous soil conditions or areas subject to inundation or ponding.

- Limit the house detail to the footprint on the grade and retaining walls

- Show, or reference the site plan for, impervious surfaces, parking, and fences.

- Label setback and right-of-way lines.

- Indicate side and rear setback lines

- Consider modifying creek setback by adding a south line that mirrors the north line.  The "creek" is then formed by a triangular area with the apex at the outlet of the culvert.

Dwg L1 – Landscape Plan

- Title - "Landscape Plan".

- Delete heading, "Topo Survey".

- Reference the arborist report.

- Describe tree removal and replacement per arborist report.

- Add landscaped area south of the patio.

- Label setback and right-of-way lines

- Indicate side and rear setback lines

- Consider modifying creek setback by adding a south line that mirrors the north line.  The "creek" is then formed by a triangular area with the apex at the outlet of the culvert.

- Per arborist report there should be 18 trees, exclusive of pine trees, to remove.  Only 14 shown.  See marked up drawing for locations.

- Removed trees should be indicated with an "X" per City requirements

- Indicate summary of proposed tree removal/planting plan

- 18 trees are recommended for removal.

- Of the 18 trees recommended for removal, 7 are in poor condition including 2 Coast Live Oaks, 2 California Bays, 1 Box Elder, 1 Willow, and 1 Bailey Acacia.

- Of the 11 remaining "good" trees recommended for removal, 8 are Coast Live Oaks and 3 are California Bays.

- An additional 6 Monterey Pines are recommended for removal because they are hazardous to life and property.

- To compensate for the removal of the 11 "good" trees we contemplate planting 24 – 15 gal replacement trees or their equivalent.

Dwg A2r – Roof Plan

- This drawing not needed (info included on site plan).

Dwg A2u – Upper Floor Plan

- Add drawing title

- North arrow

- Graphic scale

- Clarify and clean up dimension lines and eliminate extraneous, redundant,  and/or unnecessary notes

- Extend pool to column

- Add setback lines or not.  Should be consistent with ground floor plan.

Dwg A2g – Ground Floor Plan

- North arrow

- Clarify and clean up dimension lines and eliminate extraneous, redundant,  and/or unnecessary notes

- Extend pool deck to the north

- Correct legend

- Label setback lines or remove.

- Label all rooms

- Clarify upper floor areas above, e.g. driveway and entry deck

Dwg A3.1 – N and W Elevations

- Show aggregate height

- Show perpendicular height

- Show full exterior dimensions

- Indicate existing and finish grade.  If not practical, refer to cross sections.

- Indicate fencing

- Use drawing titles consistent with index

- Label the north elevation with scale

- Correct scale of west elevation

- Clarify and clean up dimension lines and eliminate extraneous, redundant,  and/or unnecessary notes

- Aluminum clad doors.

- Label roof slopes as "typ"

Dwg A3.2 – E and W Elevations

- Show aggregate height

- Show perpendicular height

- Show full exterior dimensions

- Indicate existing and finish grade.  If not practical, refer to cross sections.

- Indicate fencing

- Use drawing titles consistent with index

- Clarify and clean up dimension lines and eliminate extraneous, redundant,  and/or unnecessary notes

- Aluminum clad doors.

- Label roof slopes as "typ"

Dwg A4.1 – Section A-A   (1/4" = 1'-0")

- Use different symbol for envelope indicators.  What is shown does not match legend.

- Some of the dimensions appear incorrect or not necessary.

- Fonts on graphical scale not consistent with plan drawings

Dwg A4.2 – Section B-B  (1/4" = 1'-0")

- This large scale drawing not needed

Dwg A4.0 – Section B-B  (1/8" = 1'-0")

- Show legends

- Clarify dimensions

- Show legend

- Indicate fences

- Eliminate redundant notes and dimensions.

- Clarify height envelope

- Dwg No. should be A4.2

- Fonts on graphical scale not consistent with plans

Dwg A4.3 – Section C-C  (1/4" = 1'-0")

- Revise finished grade

- Fonts on graphical scale not consistent with plans

Dwg A4.4 – Section D-D  (1/4" = 1'-0")

- Show legends

- Show graphical scale

- Remove trees where not needed

- Fix ridge skylight

- Doesn't seem consistent with section cuts from upper and lower plans.